Nondimensional Governing Equations

We consider the steady, incompressible Navier–Stokes equations with constant fluid properties. Let 𝐿 denote the cavity wall length and 𝑈 the lid velocity. The fluid has constant density 𝜌 and dynamic viscosity 𝜇. Pressure is scaled using the dynamic pressure level.

To obtain the nondimensional form of the governing equations, the following reference scales are used:

- Length scale: 𝐿

- Velocity scale: 𝑈

- Pressure scale: 𝜌U2

Define the nondimensional variables

x= xL ,	 y= yL,	 u= uU,  v= vU,	 p= PρU2.

These substitutions are applied to the dimensional continuity and momentum equations, and constant reference factors are grouped together. For incompressible flow, density is constant and factors out of the equations.

Nondimensional Continuity Equation:

For two-dimensional incompressible flow, the continuity equation reduces to

∂u∂x + ∂x∂y = 0.

After substituting the nondimensional variables and simplifying, the nondimensional continuity equation becomes

∂u∂x + ∂v∂y = 0.

Nondimensional Momentum Equations:

Neglecting body forces and writing the steady two-dimensional momentum equations component-wise, then applying the nondimensional variables defined above, yields the following nondimensional momentum equations.

x-momentum equation

u∂u∂x + v∂u∂y = -∂p∂x + 1Re(∂2u∂x2 + ∂2u∂y2)

y-momentum equation

u∂v∂x + v∂v∂y = -∂p∂y + 1Re(∂2v∂x2 + ∂2v∂y2)

Thus, the nondimensional system consists of three equations in three unknowns: nondimensional velocities u,v and nondimensional pressure p.

Nondimensional Number and Physical Interpretation:

The only nondimensional parameter that appears in the nondimensional momentum equations is the Reynolds number,

Re = ρULμ

The Reynolds number represents the ratio of inertial forces to viscous forces in the flow.

When the Reynolds number becomes very large, viscous effects are small compared to inertial effects, and the viscous diffusion terms become negligible. In this limit, the momentum equations approach the inviscid Euler form.

When the Reynolds number becomes very small, viscous effects dominate, and inertial terms are weak. In this regime, the equations reduce toward the Stokes or creeping flow approximation.